Checking insulation thickness (2015-06-11) – H16EC0188B

Measurements by E. Cavanna and L. G. Fajardo

Data processing by E. Cavanna

Cable info:

MQXFS Coil #202

Strand material: Nb3Sn PIT

Run identification: 142A

Calibration curve

Calibration aluminium bar.

Dimensions (width x length x height): 19 x 120 x 13.95 mm.

Table 1: Detailed results S1

|  | Average displacement of detectors (mm) | Stress (MPa) | 10-stack thickness (mm) |
| --- | --- | --- | --- |
| S1-UI | -4,339 | 4,999 | 18,282 |
|  | -4,314 | 5,002 | 18,257 |
|  | -4,292 | 5,003 | 18,235 |
| Average | -4,315 | 5,001 | 18,258 |
| S1-UB | -1,322 | 4,996 | 15,266 |
|  | -1,321 | 4,996 | 15,264 |
|  | -1,320 | 5,006 | 15,263 |
| Average | -1,321 | 4,999 | 15,264 |

Table 2: Detailed results S2

|  | Average displacement of detectors (mm) | Stress (MPa) | 10-stack thickness (mm) |
| --- | --- | --- | --- |
| S2-UI | -4,337 | 4,998 | 18,281 |
|  | -4,309 | 5,007 | 18,252 |
|  | -4,295 | 5,001 | 18,238 |
| Average | -4,314 | 5,002 | 18,257 |
| S2-UB | -1,317 | 4,999 | 15,260 |
|  | -1,314 | 4,998 | 15,258 |
|  | -1,314 | 4,995 | 15,257 |
| Average | -1,315 | 4,998 | 15,258 |

Table 3: Detailed results S3

|  | Average displacement of detectors (mm) | Stress (MPa) | 10-stack thickness (mm) |
| --- | --- | --- | --- |
| S3-UI | -4,315 | 5,008 | 18,258 |
|  | -4,288 | 4,995 | 18,231 |
|  | -4,269 | 4,994 | 18,213 |
| Average | -4,291 | 4,999 | 18,234 |
| S3-UB | -1,319 | 4,999 | 15,262 |
|  | -1,315 | 5,000 | 15,259 |
|  | -1,315 | 5,000 | 15,258 |
| Average | -1,316 | 5,000 | 15,259 |

## Summary

|  | Insulated cable thickness (mm) |  | Bare cable thickness (mm) | Insulation thickness (μm) |
| --- | --- | --- | --- | --- |
| S1-UI | 1,826 | S1-UB | 1,526 | 150 |
| S2-UI | 1,826 | S2-UB | 1,526 | 150 |
| S3-UI | 1,823 | S3-UB | 1,526 | 149 |